AG-512B Coupling, Fastener, and Guard Inspection

Inspection CFI-AG512B-2026-06 | Completed 2026-06-22

# 1. Current coupling identity

| record_id | parameter | observed value | evidence | status |
| --- | --- | --- | --- | --- |
| CFI-001 | Coupling model | TorsionFlex TF-290-D | Hub marking and WO-512-02 | CURRENT |
| CFI-002 | Motor-side hub | TF290-M120-L | Laser etching | CURRENT |
| CFI-003 | Gearbox-side hub | TF290-G95-R | Laser etching | CURRENT |
| CFI-004 | Elastomer/disc element | TF-290-E95 | Package label and delivery record | CURRENT |
| CFI-005 | Spacer length | 196.0 mm | Caliper CPG-CAL-06 | CURRENT |

# 2. Bolt and locking-device inspection

| record_id | bolt | size and grade | required torque | measured torque | locking device | witness mark | result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CB-01 | 1 | M24 x 3.0, 10.9 | 790-830 N*m | 805 N*m | Installed | Present | PASS |
| CB-02 | 2 | M24 x 3.0, 10.9 | 790-830 N*m | 812 N*m | Installed | Present | PASS |
| CB-03 | 3 | M24 x 3.0, 10.9 | 790-830 N*m | 788 N*m | Installed | Present | FAIL |
| CB-04 | 4 | M24 x 3.0, 10.9 | 790-830 N*m | 830 N*m | Installed | Present | PASS |
| CB-05 | 5 | M24 x 3.0, 10.9 | 790-830 N*m | 818 N*m | Installed | Present | PASS |
| CB-06 | 6 | M24 x 3.0, 10.9 | 790-830 N*m | 801 N*m | Installed | Present | PASS |
| CB-07 | 7 | M24 x 3.0, 10.9 | 790-830 N*m | 806 N*m | Installed | Present | PASS |
| CB-08 | 8 | M24 x 3.0, 10.9 | 790-830 N*m | 799 N*m | Installed | Missing | FAIL witness mark |

# 3. Coupling guard

| record_id | check | measured or observed value | local requirement | result |
| --- | --- | --- | --- | --- |
| GRD-01 | Minimum coupling-guard clearance | 14.8 mm | Minimum 12.0 mm | PASS |
| GF-01 | Guard fasteners installed | 10 of 10 | 10 required | PASS |
| ACC-01 | Coupling-bolt access | Remove upper split-guard half | Access condition | CONTEXT |

# 4. Preliminary and commercial references

A preliminary stores sheet listed coupling FG-250, spacer 174 mm, and M20 grade 8.8 fasteners. Those entries are superseded by current field identity and completed closeout evidence.