AG-512B Guard, LOTO, Residual-Energy, and Access Checklist

Controlled checklist LOTO-AG512B Rev 7 | Verification 2026-06-24

# 1. Current twelve-point isolation list

| record_id | isolation point | energy class |
| --- | --- | --- |
| LOTO-01 | QF-512B motor disconnect | Electrical |
| LOTO-02 | VFD-512B line-side isolator | Electrical |
| LOTO-03 | VFD-512B DC-bus discharge verification | Stored electrical |
| LOTO-04 | PV-AIR-512B seal pneumatic isolation | Pneumatic |
| LOTO-05 | HV-SEAL-512B seal-oil isolation | Hydraulic |
| LOTO-06 | VV-VSL-512B vessel pressure isolation | Vessel pressure |
| LOTO-07 | BRK-SHAFT-512B shaft rotation restraint | Stored rotation |
| LOTO-08 | BV-SEAL-512B trapped seal-pressure bleed | Trapped pressure |
| LOTO-09 | SB-512B agitator support restraint | Gravitational |
| LOTO-10 | CPG-512B coupling guard lock | Mechanical |
| LOTO-11 | LT-512B local start inhibit | Electrical control |
| LOTO-12 | VFD-512B remote-start permissive isolation | Control energy |

# 2. Points omitted from the older six-point register

| record_id | omitted isolation point | energy class |
| --- | --- | --- |
| OMIT-01 | VFD-512B DC-bus discharge verification | Stored electrical |
| OMIT-02 | PV-AIR-512B seal pneumatic isolation | Pneumatic |
| OMIT-03 | HV-SEAL-512B seal-oil isolation | Hydraulic |
| OMIT-04 | VV-VSL-512B vessel pressure isolation | Vessel pressure |
| OMIT-05 | BRK-SHAFT-512B shaft rotation restraint | Stored rotation |
| OMIT-06 | BV-SEAL-512B trapped seal-pressure bleed | Trapped pressure |

# 3. Residual-energy verification

| record_id | check location | measured value | units | requirement | evidence type | result |
| --- | --- | --- | --- | --- | --- | --- |
| LOTO-RES-01 | VFD DC bus | 4.8 | V dc | <= 5.0 | Numerical measurement | PASS |
| LOTO-RES-02 | Pneumatic seal supply | 0.06 | MPa | <= 0.05 | Numerical measurement | FAIL |
| LOTO-RES-03 | Reactor vessel pressure | 0.00 | MPa | <= 0.00 | Numerical measurement | PASS |
| LOTO-RES-04 | Agitator shaft rotation | 0.0 | rpm | <= 0.0 | Laser-tach measurement | PASS |
| LOTO-RES-05 | Agitator shaft rotation |  |  | No visible rotation | Checklist attestation | CONTEXT |

# 4. Guarding and access

| record_id | item | observation | requirement | status |
| --- | --- | --- | --- | --- |
| IG-01 | Coupling guard minimum clearance | 14.8 mm | Minimum 12.0 mm | PASS |
| GF-01 | Coupling guard fasteners | 10 of 10 | 10 required | PASS |
| ACC-01 | Coupling bolt access | Remove upper split-guard half | Access condition | CONTEXT |
| ACC-02 | Seal pneumatic valve access | Remove west service panel | Access condition | CONTEXT |
| ACC-03 | Gearbox inspection port access | Open north gearbox cover | Access condition | CONTEXT |