AG-512B Reactor-Agitator Drive Service Closeout

Work package WO-AG512B-2026-06 | Closeout date 2026-06-24

# 1. Completed component changes

| record_id | component | closeout statement | status | evidence |
| --- | --- | --- | --- | --- |
| WO-512-01 | GBX-512B | Installed RexMec GXH-142-23.75 gearbox; nameplate ratio 23.75:1 | CLOSED | NFS-010; NFS-011 |
| WO-512-02 | CPG-512B | Installed TorsionFlex TF-290-D coupling, TF-290-E95 insert, and 196.0 mm spacer | CLOSED | NFS-013; NFS-014 |
| WO-512-03 | BRG set | Installed current C3-clearance motor, gearbox, and agitator bearing set | CLOSED | NFS-020:NFS-025 |
| WO-512-04 | SEAL-512B | Installed MSK-DP54-120-FKM dual pressurized Plan 54 seal kit | CLOSED | NFS-016 |
| WO-512-05 | GBX-512B | Drained mineral EP oil, flushed housing, and filled SynLube PAG ISO VG 320 | CLOSED | NFS-015 |
| WO-512-06 | GRD-512B | Installed split stainless coupling guard and ten M12 A4 fasteners | CLOSED | GRD-01; GF-01 |

# 2. Coupling bolt torque

| record_id | bolt | specification | measured torque | units | detailed result | detail |
| --- | --- | --- | --- | --- | --- | --- |
| CB-01 | 1 | M24 x 3.0 grade 10.9 | 805 | N*m | PASS | Locking plate installed; witness mark present |
| CB-02 | 2 | M24 x 3.0 grade 10.9 | 812 | N*m | PASS | Locking plate installed; witness mark present |
| CB-03 | 3 | M24 x 3.0 grade 10.9 | 788 | N*m | FAIL | Locking plate installed; witness mark present |
| CB-04 | 4 | M24 x 3.0 grade 10.9 | 830 | N*m | PASS | Equality with upper limit; witness mark present |
| CB-05 | 5 | M24 x 3.0 grade 10.9 | 818 | N*m | PASS | Locking plate installed; witness mark present |
| CB-06 | 6 | M24 x 3.0 grade 10.9 | 801 | N*m | PASS | Locking plate installed; witness mark present |
| CB-07 | 7 | M24 x 3.0 grade 10.9 | 806 | N*m | PASS | Locking plate installed; witness mark present |
| CB-08 | 8 | M24 x 3.0 grade 10.9 | 799 | N*m | PASS | Locking plate installed; witness mark absent |

Local inclusive torque range: 790-830 N*m. The cover summary originally stated that all eight bolts were within range; the detailed calibrated readings above control.

# 3. Locking devices, witness marks, and hubs

| record_id | item | observation | status |
| --- | --- | --- | --- |
| LD-01 | Coupling locking plates | 8 of 8 installed | VERIFIED |
| WM-01 | Final coupling witness marks | 7 of 8 present | FAIL |
| WM-02 | Bolt 8 witness mark | Missing | FAIL |
| HUB-01 | Motor-side hub | TF290-M120-L | VERIFIED |
| HUB-02 | Gearbox-side hub | TF290-G95-R | VERIFIED |
| SPC-01 | Installed spacer | 196.0 mm | VERIFIED |

# 4. Documentation gaps

The manufacturer material certificate for bearing lot BRG-L2606 was not included in the closeout package. Installed bearing identity remains corroborated by carton labels, housing tags, and the completed work order; the missing certificate is a traceability gap rather than an installed-identity gap.

No release-to-service decision or corrective-work instruction is contained in this record.

# 5. Sign-off

| role | name | date | status |
| --- | --- | --- | --- |
| Mechanical technician | L. Rivas | 2026-06-24 | Signed |
| Rotating equipment specialist | M. Okafor | 2026-06-24 | Signed |
| Reliability reviewer | P. Chen | 2026-06-25 | Reviewed |